AI NATIVE DISTRIBUTED DATACENTERS

A Technical Architecture for Sovereign, Resilient,
AI-Managed Edge Infrastructure

ANDD Network  |  Whitepaper v1.0  |  March 2026

contact@andd.network  |  andd.network

# Table of Contents

1. Executive Summary

2. The Case for Decentralization

3. ANDD Architecture Overview

4. Technical Architecture

5. Bandwidth & Connectivity Requirements

6. Energy Requirements & Mitigation Strategies

7. AI-Native Operations

8. Resilience Through Decentralization

9. Payment Architecture

10. DePIN Integration & Pressure Test

11. Opportunity Zone Alignment

12. Implementation Roadmap

13. Conclusion

# 1. Executive Summary

The global compute infrastructure serving AI, critical services, and digital economies is dangerously concentrated. Fewer than 30 hyperscale campuses in the United States process the vast majority of AI inference, financial transactions, government data, and emergency communications. This concentration creates single points of failure for national security, economic continuity, and community resilience — and it leaves underserved regions permanently behind.

The ANDD Vision A network of hundreds to thousands of sub-2MW edge datacenters, deployed across federally designated Opportunity Zones, forming an interconnected mesh fabric that is entirely managed and operated by artificial intelligence. No human operators provision servers. No human billing clerks process invoices. The AI management layer handles customer onboarding, capacity planning, workload routing, billing, compliance reporting, and predictive maintenance autonomously.

| <2MW | 100s+ | 4 | 99.99% |
| --- | --- | --- | --- |
| Per Datacenter | Edge Locations | Hardware Domains | SLA Target |

This whitepaper presents the technical architecture, infrastructure requirements, and economic model for the ANDD network. It details the four hardware domains (Compute, Storage, Connectivity, AI Accelerators), the bandwidth and energy profiles for each site, mitigation strategies for power resilience through renewables and microgrids, and the payment architecture spanning fiat subscriptions to real-time crypto micro-transactions.

Critically, it pressure-tests the ANDD model against the track record of existing Decentralized Physical Infrastructure Network (DePIN) projects — analyzing why consumer-grade approaches have struggled and how enterprise-grade, AI-native infrastructure resolves those failure modes.

| Smart City Infrastructure | AI at the Edge |
| --- | --- |

# 2. The Case for Decentralization

## 2.1 The Concentration Problem

The current datacenter landscape in the United States is defined by extreme geographic concentration. Northern Virginia alone hosts over 70% of internet traffic on the East Coast. The top five metro markets (Northern Virginia, Dallas-Fort Worth, Phoenix, Chicago, Silicon Valley) account for a disproportionate share of total U.S. datacenter capacity.

Global network infrastructure — concentrated and vulnerable

This concentration creates compounding vulnerabilities:

National Security Exposure. A coordinated attack on fewer than a dozen campuses could degrade federal agencies, financial markets, and defense communications simultaneously. Geographic clustering means a single regional disaster threatens a nationally significant share of compute capacity.

Latency Inequality. Communities more than 50ms from the nearest hyperscale region cannot participate in latency-sensitive applications: real-time AI inference, telemedicine, autonomous vehicle coordination, AR/VR. The digital divide is not merely a bandwidth problem; it is a proximity problem.

Sovereignty Erosion. Government data, municipal systems, and critical infrastructure become dependent on the operational decisions and pricing power of three to four hyperscale vendors.

Grid Strain. A single 300MW campus demands the equivalent output of a small natural gas plant. Dominion Energy in Virginia has cited datacenter load growth as the primary driver of new generation capacity.

## 2.2 Why Distributed Edge, Why Now

Three converging trends make the distributed edge datacenter model not only viable but urgent:

AI Inference is Moving to the Edge While training remains centralized, inference — 60-70% of all AI compute — is increasingly latency-sensitive and geographically distributed. Autonomous vehicles need sub-10ms inference. Smart-city sensors generate data that loses value traversing 2,000 miles to a hyperscale region.

DePIN Requires Enterprise-Grade Infrastructure Projects built on consumer hardware have hit reliability ceilings. Enterprise customers require 99.99% uptime, N+1 redundancy, physical security, and compliance certifications that consumer-grade nodes cannot provide.

Federal Policy Now Incentivizes Distribution The Opportunity Zone program (8,764 designated census tracts), bipartisan AI infrastructure interest, and rural broadband initiatives create an unprecedented policy tailwind.

## 2.3 The AI-Native Differentiator

Previous attempts at distributed datacenter networks failed primarily on operational economics. A sub-2MW facility cannot justify full-time site managers, network engineers, and facilities technicians. ANDD inverts this equation by making AI the operator. When all operations are handled by autonomous AI systems, the marginal cost of adding a new site approaches the cost of the hardware and connectivity alone. This is the economic unlock that makes distributed viable for the first time.

# 3. ANDD Architecture Overview

## 3.1 Network Topology

The ANDD network is organized as a three-tier mesh fabric:

| Tier | Component | Power Class | Quantity | Connects To |
| --- | --- | --- | --- | --- |
| Tier 3 | AI Control Plane | Distributed | 1 (logical) | All Regional Hubs (consensus) |
| Tier 2 | Regional Hubs | 1–2MW | 3+ initially | Control Plane + 5–15 Edge Nodes + Peering Exchanges |
| Tier 1 | Edge Nodes | 200kW–500kW | 100s–1000s | Regional Hub + Adjacent Edge Nodes (mesh) + 2nd Hub (redundant) |

ANDD edge node — enterprise-grade, modular, deployable in 60–90 days

Tier 1 — Edge Nodes (200kW–500kW). The atomic unit of the network. Modular, prefabricated facilities deployable in 60-90 days. Each contains a configurable mix of compute, storage, connectivity, and AI accelerator hardware.

Tier 2 — Regional Hubs (1–2MW). Aggregation points providing higher-density compute, regional storage caching, peering interconnection, and backhaul concentration. Each serves 5–15 edge nodes.

Tier 3 — AI Control Plane (Distributed). A distributed consensus system running across all regional hubs. Handles global workload routing, billing reconciliation, capacity planning, and AI model updates. Byzantine fault-tolerant.

## 3.2 Mesh Interconnection

Every edge node maintains four types of connectivity:

| Connection Type | Medium | Bandwidth | Purpose |
| --- | --- | --- | --- |
| Primary Uplink | Fiber / Licensed Microwave | 10+ Gbps | Primary backhaul to regional hub |
| Redundant Uplink | Diverse Fiber / 5G / LEO Satellite | 1–10 Gbps | Backup path via different route |
| Mesh Peering | Direct node-to-node | 10 Gbps | Local traffic exchange and failover |
| 2nd Hub Path | Via mesh or direct | Variable | Ensures no single hub failure isolates node |

## 3.3 Single-Site Physical Layout

| System Layer | Components | Specification |
| --- | --- | --- |
| IT Equipment | Compute Racks (x4–8), AI Accelerator Racks (x2–4), Storage Racks (x2–4) | Configurable mix based on local demand |
| Network Fabric | Top-of-Rack / Leaf-Spine Switches | 100GbE / 400GbE internal |
| Network Edge | Edge Router / Firewall | BGP peering, SD-WAN, encryption |
| External Links | Primary + Redundant Uplinks | Fiber 10G+ primary; 5G/Satellite backup |
| Power | Utility + ATS + Li-Ion UPS + Optional Solar/Fuel Cell | 15–30 min UPS runtime; microgrid capable |
| Cooling | Direct Liquid Cooling + Ambient Air Economization | Target PUE 1.15–1.25 |

# 4. Technical Architecture

## 4.1 Four Hardware Domains

### 4.1.1 Compute

| Component | Specification |
| --- | --- |
| Platform | High-density 1U/2U servers, ARM and x86 |
| Processors | AMD EPYC 9004 / Intel Xeon 6 / Ampere Altra Max |
| Memory | 256GB–1TB DDR5 per node |
| Density | 20–40kW per rack |
| Use Cases | Containerized workloads, VM hosting, CI/CD, general-purpose |

### 4.1.2 Storage

| Component | Specification |
| --- | --- |
| Hot Tier | NVMe SSD arrays, 100TB–500TB per site |
| Warm Tier | QLC SSD / high-density HDD, 500TB–2PB per site |
| Protocol | S3-compatible object, NFS/SMB block, distributed file system |
| Replication | Erasure coding across 3+ nodes; 11 nines durability |
| Use Cases | CDN caching, data lakes, AI training data, backup |

### 4.1.3 Connectivity

| Component | Specification |
| --- | --- |
| Internal Fabric | 100GbE leaf-spine, upgradable to 400GbE |
| Backhaul | 10–100 Gbps fiber (primary), diverse redundant path |
| Peering | Direct peering with regional ISPs, CDNs, cloud on-ramps |
| Wireless | 5G/LTE-A small cells, CBRS spectrum |
| Specialized | Defense network interconnection (SIPRNet/NIPRNet capable) |

### 4.1.4 AI Hardware

Purpose-built AI accelerators power inference at the edge

| Accelerator | TDP | Memory | Memory BW | Deployment |
| --- | --- | --- | --- | --- |
| NVIDIA L4 | 72W | 24GB GDDR6 | 300 GB/s | Edge Node (efficient inference) |
| NVIDIA L40S | 300W | 48GB GDDR6X | 864 GB/s | Edge Node (inference) |
| AMD MI300X | 750W | 192GB HBM3 | 5.3 TB/s | Edge Node / Hub (inference) |
| AMD MI325X | 750W | 256GB HBM3e | 6.0 TB/s | Edge / Hub (large models) |
| Intel Gaudi 3 | ~600W | 128GB HBM2e | 3.7 TB/s | Edge (cost-efficient) |
| NVIDIA H100 SXM | 700W | 80GB HBM3 | 3.35 TB/s | Regional Hub (training) |
| NVIDIA H200 SXM | 700W | 141GB HBM3e | 4.8 TB/s | Regional Hub (large models) |
| NVIDIA B200 | 1,000W | 192GB HBM3e | 8.0 TB/s | Regional Hub (training) |
| NVIDIA GB200 NVL2 | 2,700W | 384GB HBM3e | 16 TB/s | Hub (liquid-cooled) |

Edge AI Workload Feasibility:

| Workload | Hardware | Power | Latency |
| --- | --- | --- | --- |
| LLM inference (7–13B) | 1x L40S or MI300X | 300–750W | 20–100ms/token |
| LLM inference (70B) | 2–4x H100 PCIe | 700W–1.4kW | 50–200ms/token |
| Real-time image/video | 1x L4 | 72W | 5–20ms |
| Speech recognition | 1x L4 | 72W | <100ms |
| Recommender systems | 2–4x GPU + CPU | 1–3kW | 10–50ms |
| RAG | 1–2x H100 + vector DB | 1–2kW | 100–500ms |
| Fraud detection | 1x L40S | 300W | <50ms |
| Medical imaging | 1x L40S | 300W | <1s |

## 4.2 Software Stack

| Layer | Component 1 | Component 2 | Component 3 |
| --- | --- | --- | --- |
| AI Management | Onboarding Agent | Provisioning Agent | Billing + SLA Monitor |
| Orchestration | Kubernetes (K3s/RKE2) | Service Mesh (Istio) | AI-Optimized Scheduler |
| Infrastructure | SDN (OVS/VPP) | Storage (Ceph/MinIO) | DCIM/BMS (AI-Controlled) |
| Hardware Abstraction | BMC/IPMI/Redfish | GPU Driver Layer | Switch OS (SONiC/Cumulus) |

## 4.3 Security Architecture

Zero-trust networking. Every workload-to-workload communication is mTLS encrypted. No implicit trust based on network location.

Hardware root of trust. TPM 2.0 on all servers. Measured boot. Remote attestation verifies node integrity cryptographically.

Tenant isolation. SR-IOV for network, SEV-SNP/TDX for confidential computing, separate encryption keys per tenant.

Physical security. Biometric access, 24/7 AI-monitored surveillance, tamper-evident racks. SCIF-compatible options.

Compliance. FedRAMP, SOC 2, HIPAA, CJIS, ITAR compliance paths built in from day one.

# 5. Bandwidth & Connectivity Requirements

| Site Tier | Primary Uplink | Redundant | Mesh | Total Backhaul |
| --- | --- | --- | --- | --- |
| Edge Node (200kW) | 10 Gbps | 1–10 Gbps | 10 Gbps | 20–30 Gbps |
| Edge Node (500kW) | 40 Gbps | 10 Gbps | 25 Gbps | 50–75 Gbps |
| Regional Hub (1–2MW) | 100 Gbps | 40 Gbps | 100 Gbps | 200–300 Gbps |

Multi-carrier connectivity ensures resilient backhaul

### Backhaul by Site Type

| Site Type | Primary | Bandwidth | Backup | Backup BW |
| --- | --- | --- | --- | --- |
| Urban | Dark/Lit Fiber (multi-carrier) | 10–40 Gbps | 5G/LTE-A | 1–5 Gbps |
| Suburban | Fiber (1–2 carriers) | 10–40 Gbps | Diverse Fiber | 10 Gbps |
| Rural | Licensed Wireless / LEO Sat | 1–10 Gbps | 5G/LTE-A | 500M–1 Gbps |

### Protocol Stack

Underlay: eBPF-accelerated IP routing with segment routing (SRv6)

Overlay: WireGuard/MACsec encrypted tunnels; VXLAN/GENEVE tenant isolation

Peering: Automated BGP with RPKI validation and route leak detection

DNS: Anycast DNS across all nodes for automatic geo-routing

Load Balancing: AI-driven GSLB factoring latency, capacity, energy cost, carbon intensity

# 6. Energy Requirements & Mitigation Strategies

On-site solar generation offsets 30–50% of facility energy consumption

## 6.1 Per-Site Power Profile

| Site Tier | IT Load | Cooling | Infra | Total | Target PUE |
| --- | --- | --- | --- | --- | --- |
| Edge Node (200kW) | 150kW | 25kW | 25kW | 200kW | 1.15–1.25 |
| Edge Node (500kW) | 380kW | 65kW | 55kW | 500kW | 1.15–1.25 |
| Regional Hub (2MW) | 1,500kW | 280kW | 220kW | 2,000kW | 1.15–1.25 |

PUE Benchmarks — ANDD vs. Industry:

| Facility Type | Typical PUE | Best-in-Class | Primary Cooling |
| --- | --- | --- | --- |
| Hyperscale (Google, Meta) | 1.10–1.20 | 1.06–1.08 | Evaporative + liquid |
| Large colocation | 1.30–1.50 | 1.15–1.25 | Chilled water + air |
| Traditional edge (<1MW) | 1.40–1.80 | 1.20–1.30 | DX air conditioning |
| ANDD Edge (purpose-built) | 1.15–1.25 | 1.02–1.10 | DLC + immersion |

## 6.2 Cooling Technology Comparison

Direct liquid cooling eliminates the traditional edge efficiency penalty

| Technology | Cooling Power (500kW site) | Effective IT | PUE | Capital Premium |
| --- | --- | --- | --- | --- |
| Air cooling (DX) | 150–200kW | 300–350kW | 1.40–1.80 | Baseline |
| Rear-door heat exchangers | 75–125kW | 375–425kW | 1.20–1.40 | +15–25% |
| Direct-to-chip liquid | 40–75kW | 425–460kW | 1.10–1.20 | +25–40% |
| Single-phase immersion | 15–30kW | 470–485kW | 1.02–1.06 | +30–50% |
| Two-phase immersion | 10–20kW | 480–490kW | 1.01–1.04 | +40–60% |

Key Insight Moving from air cooling (PUE 1.5) to immersion (PUE 1.05) saves ~300kW of cooling power in a 1MW facility — effectively increasing usable compute capacity by 30% within the same power envelope.

## 6.3 Renewable Integration Economics

Sizing Example — 200kW Edge Node:

| Component | Specification | Cost | Annual Value |
| --- | --- | --- | --- |
| Solar Array | 100kW, carport-mounted | ~$150,000 | 175 MWh/yr generation |
| BESS (LFP) | 500kWh / 250kW | ~$200,000 | 2hr backup + daily cycling |
| IRA Tax Credit | 30–50% ITC | –$105K to –$175K | Reduces effective capital |
| Utility Savings | Offset + peak shaving | — | $40K–$80K/yr |
| Grid Services | Freq. regulation, DR | — | $50K–$200K/yr per MW |

Effective payback: 2–4 years after IRA credits and grid services revenue.

## 6.4 Federal Funding Landscape

| Program | Funding | Relevance to ANDD |
| --- | --- | --- |
| DOE GRIP | $10.5 billion | Microgrids, DER, grid modernization |
| DOE Loan Programs Office | $400B+ lending | Clean energy, fuel cells, distributed generation |
| ARPA-E COOLERCHIPS | $42 million | Datacenter cooling efficiency R&D |
| IRA Investment Tax Credit | 30–50% | Solar, battery, fuel cells at ANDD sites |
| USDA REAP | Grants + loans | Rural renewable energy |
| NTIA BEAD | $42.45 billion | Broadband + co-located edge infrastructure |

# 7. AI-Native Operations

AI agents manage the entire datacenter lifecycle autonomously

## 7.1 The Autonomous Datacenter Thesis

Traditional datacenter operations depend on human operators at every layer. These human costs establish a minimum viable facility size — typically 5MW+ — below which operations are not economically sustainable. ANDD eliminates this constraint by replacing human operational roles with specialized AI agents.

## 7.2 AI Agent Architecture

| Agent | Function | Key Capabilities |
| --- | --- | --- |
| Orchestration Agent | Coordinates all sub-agents | Conflict resolution, human escalation when confidence < threshold |
| Customer Onboarding | Full customer lifecycle | Lead qualification, proposals, KYC/KYB, tenant provisioning |
| Provisioning Agent | Infrastructure deployment | Workload-to-resource mapping, optimal node placement, auto-scaling |
| Billing & Revenue | Financial operations | Sub-second metering, invoicing, micro-transactions, fiat+crypto reconciliation |
| Predictive Maintenance | Prevent failures | Sensor telemetry, digital twins, preventive scheduling, work order dispatch |
| SLA Guardian | Ensure commitments | Threshold monitoring, auto-remediation, compliance reports, credit issuance |

Target: 1 Human Operator per 20–50 Edge Nodes A 10–25x improvement over traditional staffing models. Humans handle exception escalation, strategic decisions, physical maintenance, and compliance oversight. The AI handles everything else.

# 8. Resilience Through Decentralization

The United States’ critical infrastructure — payments, energy, communications, emergency services — has migrated to cloud systems hosted in concentrated datacenter corridors. This creates systemic risk that individual redundancy measures cannot address.

## Commercial Applications: What ANDD Protects

| Payments & Fintech | Emergency Services |
| --- | --- |

| Healthcare & Telemedicine | Government & Defense |
| --- | --- |

### 8.1 Payments Infrastructure

The U.S. payment system processes over $10 trillion daily through networks that depend on a handful of datacenter locations. ANDD distributes payment processing — card authorization, ACH, FedNow gateways — across hundreds of edge nodes with local store-and-forward capability during backbone disruptions.

### 8.2 Energy Grid

ANDD nodes host distributed SCADA/EMS systems, provide sub-10ms control loops for DERs, aggregate into a virtual power plant for grid services, and island during grid emergencies to maintain critical local services.

### 8.3 Emergency Services & Public Safety

Local NG911 hosting, FirstNet edge compute, emergency operations center computing, satellite backhaul resilience, and community notification systems — all functioning even when terrestrial backhaul is disrupted.

### 8.4 Utilities & Water Systems

Local SCADA hosting for water treatment, wastewater, and gas distribution. Semi-air-gapped operation capability. AI-powered anomaly detection for contamination, leaks, and cyber intrusion at sub-second response times.

## 8.5 Resilience Quantification

| Architecture | Uptime per Unit | System Availability | Failure Mode |
| --- | --- | --- | --- |
| Single hyperscale campus | 99.99% | 99.99% (67 min downtime/yr) | Catastrophic — total loss on failure |
| ANDD mesh (100 nodes, 70% needed) | 99.95% per node | 99.9999%+ network-level | Graceful degradation — linear performance decrease |

# 9. Payment Architecture

From enterprise wire transfers to real-time crypto micro-transactions

## 9.1 Fiat Subscription Tiers

| Tier | Model | Billing | Payment Methods | Use Case |
| --- | --- | --- | --- | --- |
| Starter | Monthly | Post-paid, NET 30 | Credit card, ACH | Startups, developers |
| Professional | Monthly/Annual | Post-paid, NET 30 | ACH, wire | Mid-market, fintech |
| Enterprise | Annual | Custom, NET 45–60 | Wire, ACH, PO-backed | Large enterprise, telcos |
| Government | Annual/Multi-year | Per procurement regs | Treasury/PO systems | Federal, state, local gov |

## 9.2 Digital & Crypto Payment Rails

| Chain | Use Case | Finality | Cost per Tx |
| --- | --- | --- | --- |
| Ethereum L1 | Large enterprise contracts | ~12 min | $1–$20 |
| Solana | High-frequency, lower-value | Sub-second | ~$0.001 |
| Ethereum L2 (Arbitrum, Base) | Medium-frequency | Minutes | $0.01–$0.10 |
| Payment Channels (off-chain) | Micro-transactions | Instant (off-chain) | Near-zero |

## 9.3 Smart Contract SLAs

ANDD introduces programmable SLAs — smart contracts that encode service level commitments and automatically enforce them:

| Contract Element | Logic | Purpose |
| --- | --- | --- |
| Uptime Target | 9999 basis points (99.99%) | SLA threshold |
| Latency Target | <5ms edge latency | Performance floor |
| Measurement Period | 30-day rolling window | Evaluation cadence |
| Oracle Integration | getUptime(tenantId, period) | Verified performance data |
| Automated Payout | paymentToken.transfer(tenant, credit) | Instant SLA credit to tenant wallet |
| Event Logging | emit SLACreditIssued() | Immutable on-chain audit trail |

## 9.4 Hybrid Payment Orchestration

| Customer Type | Primary Payment | Secondary | Micro-Tx |
| --- | --- | --- | --- |
| Enterprise | Wire / ACH (annual) | Credit card (overage) | N/A |
| Government | PO / Treasury | ACH | N/A |
| Startup | Credit card (monthly) | USDC | Per-API-call |
| DePIN Project | Stablecoin | Payment channel | Per-request |
| Web3 Native | USDC / token | Payment channel | Per-request |

# 10. DePIN Integration & Pressure Test

DePIN — the promise of decentralized physical infrastructure meets enterprise reality

## 10.1 Case Studies: Lessons from DePIN Projects

| Project | What Worked | What Failed | ANDD Lesson |
| --- | --- | --- | --- |
| Helium | 900K+ hotspots globally; proved token incentives bootstrap networks | Over-provisioned where enthusiasts live, not where demand is; consumer hardware = no SLA | Location must be demand-driven; enterprise hardware required |
| Filecoin | 22+ EiB storage; cryptographic proof-of-storage | Mostly empty "committed capacity"; favors industrial miners; slow retrieval | Incentives tied to utilization, not capacity |
| Render | Real 3D rendering demand; millions of jobs processed | Consumer GPUs can't guarantee uptime, thermal mgmt, or ECC for AI inference | AI compute must be datacenter-grade |
| Akash | 70–85% cheaper than hyperscale; functioning marketplace | Inconsistent uptime; no SLA enforcement; enterprise won't trust anonymous providers | Smart contract SLAs + enterprise facilities = trust layer |
| io.net | Distributed GPU clustering for AI/ML | Heterogeneous consumer GPUs; inter-node latency kills training; benchmark ≠ production | Homogeneous, co-located hardware within each site |

## 10.2 The Enterprise-Grade Gap

| Dimension | Consumer DePIN | ANDD |
| --- | --- | --- |
| Uptime | 90–98% (best effort) | 99.99% (SLA-backed) |
| Redundancy | None | N+1 power, cooling, network |
| Physical Security | None | Biometric, surveillance, tamper-evident |
| Compliance | None | FedRAMP, SOC 2, HIPAA, CJIS |
| Power Quality | Residential (variable) | UPS + BESS + generator/fuel cell |
| Cooling | Consumer fans | DLC, immersion, AI-optimized |
| Connectivity | Residential broadband | Multi-provider, redundant, SLA-backed |
| Data Sovereignty | Unknown | Configurable per-tenant geo-restrictions |

ANDD Completes DePIN ANDD does not compete with DePIN projects — it completes them. The tokenomics and demand aggregation work; the physical infrastructure layer needs upgrading to enterprise grade. ANDD provides that backbone.

## 10.3 What Can Be Built on ANDD

ANDD as infrastructure enables an ecosystem of applications and services that require reliable, distributed, low-latency compute:

| Smart City & IoT | Telemedicine & Health AI |
| --- | --- |

| Community Connectivity | DePIN & Web3 Protocols |
| --- | --- |

| Application Category | Examples | Why ANDD is Required |
| --- | --- | --- |
| Smart City & IoT | Traffic management, environmental sensors, connected streetlights, digital twins | Sub-10ms latency for real-time control; local data processing |
| Autonomous Vehicles | V2X communication, HD map updates, real-time inference | Safety-critical latency requirements; geographic coverage |
| Telemedicine & Health AI | Remote diagnostics, medical image analysis, patient monitoring | Data residency (HIPAA), low latency, high availability |
| Edge AI / MLOps | Model serving, fine-tuning, RAG pipelines, AI agents | GPU at the edge; avoid round-trip to hyperscale |
| DePIN Protocols | Decentralized compute, storage, wireless, sensing networks | Enterprise-grade backbone for consumer-proven protocols |
| Fintech & Payments | Real-time payment processing, fraud detection, crypto settlement | Data residency, sub-50ms latency, regulatory compliance |
| Content Delivery | Video streaming, gaming, AR/VR content, CDN edge caching | Bandwidth offload, latency reduction |
| Defense & Intelligence | Classified workloads, SIGINT processing, C2 systems | Sovereign infrastructure, defense network compatible |
| Emergency Services | NG911, FirstNet, incident management, community alerts | Must function during disasters when backhaul fails |
| Agricultural Tech | Precision agriculture, drone coordination, sensor networks | Rural coverage, local processing, low-bandwidth backhaul |

# 11. Opportunity Zone Alignment

Building digital infrastructure in the communities that need it most

## 11.1 Tax Incentive Structure

| Tier | Incentive | Requirement | Benefit |
| --- | --- | --- | --- |
| Tier 1 | Capital Gains Deferral | Reinvest into QOF | Defer tax on eligible gains; rolling 5-year window |
| Tier 2 | Basis Step-Up | Hold 5+ years | 10% step-up (30% for rural OZ via Qualified Rural OFs) |
| Tier 3 | 10-Year Exclusion | Hold 10+ years | Zero federal capital gains tax on all new appreciation |

## 11.2 Per-Site Community Impact

| Impact Category | Estimate (200kW Edge Node) |
| --- | --- |
| Construction jobs | 15–30 (6–12 month build) |
| Permanent operations jobs | 1–3 (technicians, supplemented by AI) |
| Annual property tax revenue | $50,000–$150,000 |
| Annual utility revenue | $150,000–$300,000 |
| Indirect economic multiplier | 1.5–2.0x direct spending |
| Digital infrastructure | Gigabit connectivity, AI capabilities, edge compute |

# 12. Implementation Roadmap

| Phase | Timeline | Key Milestones |
| --- | --- | --- |
| Phase 1: Foundation | Months 1–12 | AI management layer; 200kW reference design; 3–5 pilot sites; SOC 2 Type I; QOF formation |
| Phase 2: Network Formation | Months 12–24 | 20–50 nodes; regional hubs; >90% autonomous ops; first gov customers; DePIN APIs; FedRAMP |
| Phase 3: Scale | Months 24–48 | 100–300 nodes; smart contract SLAs; micro-transactions; federal customers; defense integration |
| Phase 4: Continental Coverage | Months 48–72 | 500–1,000+ nodes; full AI autonomy; international expansion; VPP grid services; self-sustaining |

# 13. Conclusion

The concentration of compute infrastructure into a handful of hyperscale campuses was an artifact of an era when operational economics demanded scale and AI did not yet exist as an operational tool. That era is ending.

ANDD demonstrates that AI-native operations eliminate the economic barrier to distributed deployment. When AI handles onboarding, provisioning, billing, monitoring, and maintenance, the marginal cost of adding a new edge node approaches the cost of the hardware itself.

ANDD delivers on every dimension that matters: • Sovereignty — Compute stays where it is generated and governed • Security — No single target can degrade the network • Resilience — Distributed power, connectivity, and compute survive events that cripple centralized alternatives • Equity — Advanced capabilities reach underserved communities • DePIN Readiness — Enterprise-grade backbone for decentralized protocols • Economic Viability — Multiple payment rails serve every customer type

The question is no longer whether distributed, AI-managed datacenter infrastructure is technically feasible. The question is how quickly it can be deployed.

ANDD is the answer.

contact@andd.network  |  andd.network

This document contains forward-looking statements about technology capabilities, market conditions, and business plans. Actual results may differ. Technical specifications represent design targets. Regulatory and tax information is for informational purposes and should not be construed as legal or tax advice.